# System Context: 1D Building Energy Model

This document provides a technical abstraction of the bem_1d.py script. It defines the software architecture, state management, and input/output schemas required to interface with or modify the BuildingEnergyModel class.

## Architecture

The script implements a discrete-time thermodynamic simulation using an explicit Euler integration scheme. It functions as a state machine. The BuildingEnergyModel class stores the physical parameters and current thermodynamic state. The step() method advances the simulation by a specified time delta (dt), ingests external forcing variables, mutates the internal state, and returns the updated state dictionary.

## State Management

The class initializes via a parameters dictionary. It maintains two categories of variables:

### Static Physical Parameters

Calculated once during __init__.

- Geometric: volume, a_roof, a_floor, a_wall_n, a_wall_s, a_wall_e, a_wall_w.

- Thermal Capacitance: internal_heat_capacity.

- Environment Variables: lat_rad, doy, rho, cp.

- Envelope Properties: env_thickness, env_density, env_cp, env_k, env_emiss, shgc, wall_albedo, roof_albedo.

- HVAC Setpoints: t_cool_target, t_heat_target, q_target, cop_cool, cop_heat.

### Dynamic State Variables

Mutated during each call to step().

- t_in: Indoor air temperature (Kelvin).

- q_in: Indoor specific humidity (kg/kg).

- t_surf: Dictionary containing prognostic surface temperatures for keys ['Roof', 'N', 'S', 'E', 'W'].

## Execution Flow: The step() Method

The step() method requires two arguments: dt (float, seconds) and forcing (dictionary).

### Forcing Input Schema

The forcing dictionary must contain the following keys:

- time_hours (float): Current time in hours (0.0 to 24.0 or cumulative).

- t_amb (float): Ambient outdoor temperature (K).

- q_amb (float): Ambient outdoor specific humidity (kg/kg).

- sw_in (float): Global horizontal irradiance (W/m^2).

- wind_speed (float): Local wind speed (m/s).

- ac_on (bool): HVAC system operational status.

- occup_sens (float): Sensible heat from occupants (W).

- equip_sens (float): Sensible heat from equipment (W).

- occup_lat (float): Latent heat from occupants (kg/s).

- vent_rate (float): Volumetric flow rate of ventilation (m^3/s).

- t_ground (float, optional): Deep ground temperature. Defaults to 295.0 K.

### Algorithmic Sequence

- Solar Geometry: Computes zenith and azimuth angles. It clamps cos_zenith to [-1.0, 1.0] and denominators to a minimum of 0.05 or 0.0001 to prevent mathematical domain errors and asymptotic spikes at sunrise/sunset.

- Radiation Partitioning: Splits global horizontal irradiance into direct normal and diffuse components. Calculates incident radiation on the roof and four vertical walls.

- Envelope Energy Balance: Iterates over the five surfaces. Calculates shortwave absorption, longwave emission, external convection, and internal conduction. Mutates self.t_surf.

- Indoor Sensible Balance: Aggregates internal loads, window solar transmission, wall conduction, ventilation exchange, and ground conduction. Predicts t_in_next.

- Indoor Latent Balance: Aggregates occupant latent loads and ventilation moisture exchange. Predicts q_in_next.

- HVAC Intervention: If ac_on is True and predicted states violate setpoints, the model calculates the exact q_hvac and e_hvac required to meet setpoints.

- State Mutation: Updates self.t_in and self.q_in with the final net fluxes.

### Output Schema

The method returns a dictionary of the updated states:

- t_in (float): Updated indoor temperature.

- q_in (float): Updated indoor humidity.

- sens_cool_out (float): Waste sensible heat ejected to the exterior from cooling (W). Includes compressor work.

- sens_heat_out (float): Waste sensible heat ejected to the exterior from heating (W).

- t_roof (float): Roof temperature.

- t_wall_s (float): South wall temperature.

- t_wall_n (float): North wall temperature.

- t_wall_e (float): East wall temperature.

- t_wall_w (float): West wall temperature.

## Known Constraints for Agentic Interaction

- Numerical Stability: The explicit Euler integration scheme requires a sufficiently small dt. High thermal conductivity (env_k), low envelope heat capacity, or large time steps will cause the numerical solution to diverge, resulting in extreme temperature oscillations.

- Stateless Forcing: The model has no internal memory of past forcing variables. Any diurnal or weather-based patterns must be generated externally and passed via the forcing dictionary at each step.